Week-10

DLD LAB-10

Name: shamaim noor

L1f24BSCS0040

Binary to Gray Code Conversion, Half Adder, and Full Adder Circuit

Objectives:

- To understand concept of Binary to Gray Code Conversion, Half Adder and Full Adder circuits.

- To validate implementation of circuits using Circuit Maker 2000.

| Decimal Number | 4-bit Binary Code |  |  |  | 4-bit Gray Code |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B4 | B3 | B2 | B1 | G4 | G3 | G2 | G1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 3 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 5 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 7 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 8 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 10 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 11 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 |
| 12 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 13 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 14 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 15 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

Binary to Gray Code Conversion

Binary to Gray code Conversion:

Gray to Binary code Conversion:

General Rule for Conversion:

- G₄ = B₄

- G₃ = B₄ ⊕ B₃ (XOR)

- G₂ = B₃ ⊕ B₂

- G₁ = B₂ ⊕ B₁

Example: Decimal 5

- Binary (B₄ B₃ B₂ B₁) = 0101

- Compute Gray Code:

  - G₄ = B₄ = 0

  - G₃ = B₄ ⊕ B₃ = 0 ⊕ 1 = 1

  - G₂ = B₃ ⊕ B₂ = 1 ⊕ 0 = 1

  - G₁ = B₂ ⊕ B₁ = 0 ⊕ 1 = 1

- Gray Code = 0111

Example: Decimal 10

- Binary = 1010

- G₄ = 1

- G₃ = 1 ⊕ 0 = 1

- G₂ = 0 ⊕ 1 = 1

- G₁ = 1 ⊕ 0 = 1

- Gray Code = 1111

Half Adder Circuit

In Digital Logic Design (DLD), a Half Adder is a basic combinational circuit used to add two single-bit binary numbers.

Inputs:

- A (1-bit)

- B (1-bit)

Outputs:

- Sum (S) → result of A + B

- Carry (C) → carry-out bit if there's an overflow

Logic Expressions:

- Sum (S) = A ⊕ B (XOR gate)

- Carry (C) = A · B (AND gate)

Truth Table:

| A | B | Sum (A ⊕ B) | Carry (A · B) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Circuit Diagram:

Full Adder Circuit

A Full Adder is a combinational circuit that adds three 1-bit binary numbers:

- Input A

- Input B

- Carry-in (Cin from previous stage)

It gives:

- Sum (S)

- Carry-out (Cout)

Inputs:

- A (1-bit)

- B (1-bit)

- Cin (Carry-in)

Logic Expressions:

- Sum = A ⊕ B ⊕ Cin

- Carryout = (A·B) + (Cin.(A ⊕ B)

LAB TASKS

TASK-01

Implement a 4-bit Binary to Gray Converter on Circuit Maker and verify the results using the above given table. Please provide the circuit snapshots against highlighted combinations.

Solution:

TASK-02

Implement a 4-bit Gray to Binary Converter on Circuit Maker and verify the results using the above given table. Please provide the circuit snapshots against highlighted combinations.

Solution:

TASK-03

- Draw a truth table for half adder. Clearly mention all inputs and outputs.

- For both CARRY and SUM, write standard SOP Expression.

- For both CARRY and SUM, write the Minimized SOP using K-Map grouping techniques.

- For both standard and minimized expression on Circuit Maker and verify the results.

Solution:

| A | B | Sum (S) | Carry (C) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

b) SOP expression for Sum:

Sum (S)=A′B+AB′

SOP expression for Carry:

carry (C)=AB

c) Minimized SOP for Sum:

Sum (S)= A′B+AB′

Minimized SOP for Carry:

Carry (C)= AB

d)

TASK-04

- Draw a truth table for full adder. Clearly mention all inputs and outputs.

- For both CARRY and SUM, write standard SOP Expression.

- For both CARRY and SUM, write the Minimized SOP using K-Map grouping techniques.

- For both standard and minimized expression on Circuit Maker and verify the results.

Solution:

a)

| A | B | Cin | Sum (S) | Carry (Cout) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

b)

Standard SOP expression for Sum:

Sum (S)= A′B′Cin+A′BCin′+AB′Cin′+ABCin

Standard SOP expression for Carry:

Carry (Cout)= A′BCin+AB′Cin+ABCin′+ABCin

c) Minimized SOP for Sum:

Sum (S)= A⊕B⊕Cin

Minimized SOP for Carry:

Carry (Cout)= AB+ACin+Bcin

d)

standard circuit:

Simplified form: